# Electric School Bus (ESB) Vehicle and Charger Commissioning Guide

Contents

Overview	1

Electric School Bus Commissioning Procedures Checklist	2

Before the Bus and Charger Arrive	2

When the Bus and Charger Arrive/Before Anyone Drives – Paperwork	3

When the Bus and Charger Arrive/Before Passengers are On Board – Training	4

When Bus and Charger Arrive/Before Passengers are On Board – Testing	5

When Bus and Charger Arrive/Before Passengers are On Board – Maintenance	6

Electric School Bus Vehicle and EVSE Commissioning Form	7

Appendix	11

# Overview

Commissioning is a quality assurance process of testing, verifying and documenting a new electric school bus (ESB) and its accompanying charging infrastructure to ensure it operates as intended. This guide includes a Commissioning Procedures Checklist that provides recommended commissioning activities for before and when the school bus arrives, and a Commissioning Form to be used for each vehicle and charger. Together, the checklist and form are intended to facilitate the adoption experience of new ESBs and their accompanying charging infrastructure and create a central record of important information regarding systems and electric vehicle supply equipment (EVSE).

They will help verify that upon delivery, vehicles and charging equipment:

- Meet technical specs stated by manufacturers and vendors

- Perform to standards and meet the needs of the school district

- Perform safely prior to transporting students

This checklist and form are not exhaustive but are meant to provide guidance on relevant actions to take between ordering an electric school bus and transporting students. Steps are not always sequential, and school districts should determine the best course for their own testing procedures. Elements are broken down by those that should be done per school bus and those that should be done per procurement.

As local conditions, regulations and inspections vary, this template has been provided in Word (.docx) format. Users should add or remove rows and elements as needed or pull information into existing checklists or procedures already underway in their transportation departments.

NOTE: The Appendix and in-line comments offer additional information on each consideration.

To Print the Commissioning Checklist and Form without comments, go to File > Print > then under the drop down, “Print all Pages” (click down arrow), and uncheck “Print Markup.”

For more information on electric school buses and resources to help with planning for adoption, please visit: electricschoolbusinitiative.org. If you have suggestions on how to improve this resource, please email our team at ESBinfo@wri.org.

# Electric School Bus Commissioning Procedures Checklist

## Before the School Bus and Charger Arrive

Before the school bus arrives, prepare the physical space for the new ESB and its charger and assign roles and responsibilities, including identifying who will conduct trainings upon arrival.

| Tasks | Completed | Date Completed | Name | Notes |
| --- | --- | --- | --- | --- |
| Per School Bus |  |  |  |  |
| Configure/confirm parking and charging infrastructure for ESB | ☐ |  |  |  |
| Conduct a route analysis and identify routes for the ESB(s) | ☐ |  |  |  |
| Engage first responders | ☐ |  |  |  |
| Assign roles for charging | ☐ |  | Responsible for plugging in after route Position Title: Responsible for monitoring charging Position Title: Responsible for unplugging/storing charger cable Position Title: Responsible during long breaks (summer/winter) Position Title: Responsible for managing charging to optimize charger performance and energy usage (demand charges, peak charges, time-of-use charges) Position Title: Back-up bus if ESB is not charged overnight or power outage. Bus #: |  |
| Identify warranty and service agreement coverage | ☐ |  |  |  |
| Per Procurement |  |  |  |  |
| Identify who will train ESB drivers and technicians | ☐ |  |  |  |
| Designate the trainer(s) for “Train-the-Trainer” | ☐ |  |  |  |
| EVSE: Charger and Hardware Components |  |  |  |  |
| Determine charging cord cable management system | ☐ |  |  |  |
| Determine Wi-Fi connectivity for chargers | ☐ |  |  |  |
| Incorporate IT protocols for system security | ☐ |  |  |  |

## When the School Bus and Charger Arrives/Before Anyone Drives – Paperwork

Prior to operation, get all paperwork squared away. Some of these procedures may already be in place in your transportation department for any bus regardless of fuel type (title, DMV registration) while others will have new components to consider for electric models (safety guide, first responder guide).

| Tasks | Completed | Date Completed | Name | Notes |
| --- | --- | --- | --- | --- |
| Per School Bus |  |  |  |  |
| Complete commissioning form (pg7) | ☐ |  |  |  |
| Acquire title | ☐ |  |  |  |
| DMV registration (lessee/lessor)/ plating installed | ☐ |  |  |  |
| Key handoff/key management | ☐ |  |  |  |
| Vehicle Identification Number (VIN) documentation | ☐ |  |  |  |
| Department of Transportation (DOT) inspection or registration stickers valid and on vehicle | ☐ |  |  |  |
| Certificate of inspection by state/community completed | ☐ |  |  |  |
| Certificate of insurance in vehicle | ☐ |  |  |  |
| Owner’s manual reviewed and in vehicle | ☐ |  |  |  |
| School bus labeling | ☐ |  |  |  |
| Per Procurement |  |  |  |  |
| Executed lease agreement | ☐ |  |  |  |
| Safety guide reviewed | ☐ |  |  |  |
| First responder guide developed and available | ☐ |  |  |  |
| Additional safety items (e.g., manual, stickers, warnings) | ☐ |  |  |  |

When the School Bus and Charger Arrive/Before Passengers are On Board – Training
Conducting proper training will help your drivers and technicians feel comfortable with the new school bus and fuel type before any route operations.

| Potential Trainings to Conduct | Completed | Date Completed | Name | Notes |
| --- | --- | --- | --- | --- |
| Per School Bus |  |  |  |  |
| Aggregate videos and training material | ☐ |  |  |  |
| Plugging in/ unplugging the ESB + cable storage | ☐ |  |  |  |
| Powering ESB on/off | ☐ |  |  |  |
| Regenerative braking | ☐ |  |  |  |
| Performance on hills | ☐ |  |  |  |
| Gradual acceleration techniques | ☐ |  |  |  |
| Optional equipment (e.g., radios, cameras, child check, etc.) | ☐ |  |  |  |
| Per Procurement |  |  |  |  |
| Powertrain assembly | ☐ |  |  |  |
| Radiator, air compressor | ☐ |  |  |  |
| Fluid locations and refill | ☐ |  |  |  |
| Orange high voltage cables |  |  |  |  |
| 12-volt battery switch | ☐ |  |  |  |
| High voltage battery | ☐ |  |  |  |
| Regenerative braking switch (if applicable) | ☐ |  |  |  |
| Other sub-components | ☐ |  |  |  |
| First Responder training | ☐ |  |  |  |
| EVSE: Charger and Hardware Components |  |  |  |  |
| Establish charging procedures and best practices | ☐ |  |  |  |
| Establish preventative maintenance procedures for chargers | ☐ |  |  |  |
| Electric vehicle drive system explanation and trainings received/completed | ☐ |  |  |  |

## When School Bus and Charger Arrive/Before Passengers are On Board – Testing

Before the school bus begins running daily routes with passengers, test the bus under a range of conditions to understand how its performance is impacted. Convey any issues to the dealer or manufacturer.

| Tasks | Completed | Date Completed | Name | Notes |
| --- | --- | --- | --- | --- |
| Per School Bus |  |  |  |  |
| Identify main driver(s) for ESB(s) | ☐ |  |  |  |
| Assign radio units and test function on both frequencies | ☐ |  |  |  |
| Test drive for at least 200 miles and min. of two drivers | ☐ |  |  |  |
| Identify nameplate and usable range | ☐ |  |  |  |
| Track efficiency and performance (kWh/mile) | ☐ |  |  |  |
| Set benchmark for number of problem-free days | ☐ |  |  |  |
| Initial vehicle checks (see comment for examples): | ☐ |  |  |  |
| Per Procurement |  |  |  |  |
| Establish procedures for low-/no-charge | ☐ |  |  |  |
| -In the yard | ☐ |  |  |  |
| -In the field | ☐ |  |  |  |
| EVSE: Charger and Hardware Components |  |  |  |  |
| Confirm charger/charger management software and vehicle interoperability | ☐ |  |  |  |
| Confirm charger works for seven (7) days | ☐ |  |  |  |
| Test charge time from 20% to 100% | ☐ |  |  |  |

## When School Bus and Charger Arrive/Before Passengers are On Board – Maintenance

Technicians have fewer parts to maintain for electric school buses compared with their diesel-burning counterparts. High voltage (HV) maintenance will likely be handled by the OEM/dealer for the duration of the warranty.

The vehicle manufacturer will provide recommended preventative maintenance intervals and guidance. These intervals will vary and will need to be incorporated into existing internal documents such as daily vehicle inspection reports (DVIRs) and fleet maintenance software systems.

| Tasks | Completed | Date Completed | Name | Notes |
| --- | --- | --- | --- | --- |
| Per School Bus |  |  |  |  |
| Inspection sheets | ☐ |  |  |  |
| - Daily inspection sheet | ☐ |  |  |  |
| - Periodic inspection sheet | ☐ |  |  |  |
| - Preventative maintenance checklist | ☐ |  |  |  |
| - Dealer service chart | ☐ |  |  |  |
| Regular school bus maintenance | ☐ |  |  |  |
| - Fluids, filters, windshield wipers | ☐ |  |  |  |
| - Brakes | ☐ |  |  |  |
| Install after-market equipment | ☐ |  |  |  |
| Confirm warranty contact info and procedures for service | ☐ |  |  |  |
| Per Procurement |  |  |  |  |
| Personal protective equipment location, procedures for use | ☐ |  |  |  |
| Identify and disseminate emergency and standard operating procedures | ☐ |  |  |  |
| Identify how to update operating systems | ☐ |  |  |  |
| Understand cleaning procedures | ☐ |  |  |  |
| EVSE: Charger and Hardware Components |  |  |  |  |
| Understand cleaning procedures | ☐ |  |  | Cleaning? Yes ¦ No How: |
| Power shut off | ☐ |  |  |  |
| Identify signs of wear and tear | ☐ |  |  |  |

# Electric School Bus Vehicle and Charger (EVSE) Commissioning Form

Vehicle

Today’s Date: __________   Date School Bus Received: ___________  School Bus Number: ____________

◯ # is on school bus ◯ not yet #ed

Chassis Manufacturer: __________________    Body Manufacturer: ____________________

Build Type: ◯ Newly-Manufactured  ◯ Repowered  ◯ Used

VIN: __ __ __ __ __ __ __ __ __ __ __ __ __ __ __ __ _____    Model Year:  ___ ___ ___ ___

GVWR: __________    Odometer: __________    Capacity:  _____ children or _____ adults

Vehicle Dimensions (maximums): _________ length, ___________ width, __________ height

Wheelbase: ________         Vehicle charging (accepts): ◯ L2 AC   ◯ L3 DC    ◯ Both

Charge Port Location: ◯ Front Nose/Grill    ◯ Front Driver’s Side    ◯ Rear Driver’s Side

◯ Rear                      ◯ Front Curbside          ◯ Rear Curbside

Maximum kW Input Limit (charging speed): _____________________________

Type: ◯ Type A   ◯ Type B    ◯ Type C    ◯ Type D    ◯ Other _______________

Wheelchair Lift:    ◯ no  ◯ yes		Adjustable (ADA) seating: ◯ no  ◯ yes

Other Equipment (A/C, heater [diesel or electric], Wi-Fi, cellular): _______________________________

Other After-Market Equipment (camera system, AVL): __________________________________

License Plate: ________________________  State of Registration:_______

Registration Expiration Date: _______________ Vehicle Title Owner:____________________

Vehicle is Leased:    ◯ no  ◯ yes	If leased, lessee:_________________________

If leased, lessor:____________	    Lease expiry:___________	Lease max mileage:________

State Inspection:  ◯ not required  ◯ none  ◯  yes and expiration date: ___________________

Last inspection date:___________________	Next inspection date:___________________

Defects:     ◯ no  ◯ yes ________________________________________________________

Other Inspection: ◯ not required  ◯ none  ◯  yes and expiration date: ___________________

Fleet Management Software/Telemetry Platform:  ◯ none  ◯ unknown

◯  yes, platform: ____________________  Software License Number: ___________________________

GPS data logger Serial Number:_______________     Radio:  ◯ none  ◯  yes and unit number: _______

List other Parasitic Loads (lights, cameras, internal components): ________________________________

Minimum state of charge (SOC):_____  Battery Chemistry:____________  Battery Capacity (kWh):_____

Under hood 12V Battery size, chemistry, and age: ______________________________________

Electric Vehicle Supply Equipment (EVSE) – Charger and Hardware Components

Today’s Date: _______________          Date Charger(s) Received: ________________

Manufacturer: ____________________ Model: ____________  Serial Number: ____________________

OCCP Compliance (list version): __________________   Charger Supplier: ______________________

Type (Level 2/DC): ______   Power Level (kW): _____    Plug Type (J1772/CCS/CHAdeMO): __________

Number of Charging Ports (handles): _____    ◯ Simultaneous Charging   ◯  Sequential Charging

Amperage: _______________    Input Voltage: _______________   Output Voltage: ________________

Network Connection (None/Ethernet/Wi-Fi/Fiber/Cellular): _______________

IP Address: _______________________    Modem Model: _________________

Series Compatible (Daisy-chain capable): ◯ no  ◯ yes

Type of Installation:   ◯ Wall-mounted  ◯ Pedestal-mounted   ◯ Pole-mounted  ◯ Mobile charger

LED Screen: ◯ no  ◯ yes        UV Coating for LED Screen: ◯ no  ◯ yes

Power Conditioning System/Controller:  ◯ no  ◯ yes    Manual Control Buttons: ◯ no  ◯ yes

Protective Equipment:   ◯ Bollards      ◯ Cable Management System      ◯ Handle cradle

(check all that apply)      ◯ Outdoor/NEMA housing rate    ◯ Emergency shut-off button on charger

Power Shut-Off Points:  ◯ Switch on charger    ◯ Breaker panel    ◯ Main disconnect  ◯ Other(s)  _______

Service Plan (# of months): _________   Service Technician Availability (# of hours): ____________

Guaranteed spare parts availability: ◯ no  ◯ yes

Specialized Training Requirements (i.e., DCFC High voltage service): ◯ no  ◯ yes __________________

Is this charger shared with multiple vehicles? ◯ no, dedicated to this bus  ◯ yes, how many others ____

Can this charger be networked? ◯ no  ◯ yes

If yes, is this charger equipped with managed charging capabilities?  ◯ no  ◯ yes

If yes: ◯ software included in charger   ◯ third-party managed charging platform

Name of software: ____________________  ID for vehicle in software: ________________

Includes (check all that apply):

◯ Cloud access   ◯ Customer management portal   ◯ Portal dashboard

◯ Ability to program various rates (time-of-use/pricing variations)

◯ Historical data retention   ◯ Downloadable data method (.cvs/.pdf/Word/other)

◯ Smartphone app/mobile control ◯ Remote reset capability

Customer Service Availability (hours of operation): _______________

Includes (check all that apply):

◯ Customer service issue logging

◯ Issue resolution and completion logging and closure process

◯ Root cause analysis (RCA) to prevent recurrence of the same issues

Customer Notification Method(s): ◯ Text   ◯ Email    ◯ Phone   ◯ Others: ___________

Notifications include (check all that apply):

◯ Charger availability   ◯ Charger malfunction   ◯ Charger connection

◯ Active charging   ◯ Plugged-in but not charging   ◯ Real-time KW output

◯ kWh total output   ◯ Charging session completed   ◯ Load graphing

Purchase Information

Vehicle Purchase Date: ________________  Vehicle Seller: _______________________________

Vehicle Invoice Number: ________________________

EVSE (Charger) Purchase Date: ___________  EVSE (Charger) Seller: ________________________

EVSE (Charger)  Invoice Number: ________________________

Warranty Information

Responsible Party – Battery: _________________________    Duration: ________________________

Responsible Party – 12V Battery: _____________________     Duration: ________________________

Responsible Party – Drive Train: ______________________    Duration: ________________________

Responsible Party – Chassis: _________________________    Duration: ________________________

Responsible Party – Charger: ________________________     Duration: ________________________

Responsible Party – Other: __________________________    Duration: ________________________

Attachments

◯ Title ◯ Warranty   ◯ Manual   ◯ Invoice   ◯ State Inspection Certificate   ◯ Use Requirements

◯ Replacement Plan  ◯ Charger Maintenance Recommendations   ◯ Charger Spare Parts List

◯ Single-Line Drawing for Electrical Connection   ◯Pictures of Final Installation

◯ Service Agreement (if any)

Vehicle Pictures: ◯ Front ◯ Left side ◯ Right side ◯ Rear ◯ Interior from front ◯ Interior from rear

Open Issues

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

# Appendix

This Appendix provides image examples and links to additional tools and resources that highlight the respective sections and topics.

## Electric School Bus Vehicle and EVSE Commissioning Form

Managed Charging resource page: All About Managed Charging and “Vehicle-to-Everything” or V2X

Types of EVSE installation

Wall-mounted               Pedestal-mounted                 Pole-mounted  	             Mobile Charger

## Before the School Bus and Charger Arrive

- Electric School Bus Market Study

- 8 Tips For Common Electric School Bus Charging Challenges

- Electric School Bus Technician Training Database

- Incorporate equity into route selection: How School Districts Can Include Equity When Choosing Where to Deploy Electric School Buses First

- Configure and confirm parking and charging infrastructure for ESBs: Electric School Bus Facility Assessment Guide

- Determine cable management system:

Blue arrow: cable management system. Green arrow: bollard

## When the School Bus and Charger Arrive/Before Anyone Drives – Paperwork

## Establishing a Service Level Agreement (SLA) for ESBs and chargers to address issues with operations and maintenance: All About Service Level Agreements (SLAs) for Electric School Buses and Chargers

## When the School Bus and Charger Arrive/Before Passengers on Board – Training

- Potential Trainings to Conduct: Reskilling the Workforce: Training Needs for Electric School Bus Operators and Maintenance Technicians

- Training opportunities for technicians: Electric School Bus Technician Training Database

- Establish charging procedures and best practices: How Electric School Bus Owners Can Maximize Battery Performance by Limiting Aging

- Video guides on charging basics: All About Charging Infrastructure

## When the School Bus and Charger Arrive/Before Passengers on Board – Testing

- Identify nameplate and usable range: All About Range and Reliability

## When the School Bus and Charger Arrive/Before Passengers on Board – Maintenance

- School bus: Electric School Bus Maintenance: Key Similarities, Differences

- Battery recycling: How Electric School Bus Fleet Owners Can Ensure That Their Used Vehicle Batteries Avoid the Landfill – and Reap Economic and Environmental Benefits

Photo from vecteezy.com

Heliox, Mobile 50kW Charger